Reginald True

Professor Asadi

CS 350

20 Febuary 2026

Syntec Report

SysTec wants to enter the smart thermostat market. So I built a working prototype to show that the basic thermostat functions work correctly. The prototype uses an AHT20 temperature sensor connected through I2C to read the room temperature. The default set point is 72 degrees Fahrenheit. The system compares the current temperature to the set point to decide what to do. The thermostat has three states OFF, HEAT, and COOL. A button cycles between these states. Two other buttons increase or decrease the set temperature by one degree at a time.

Two LEDs show what the thermostat is doing. The red LED represents heating, and the blue LED represents cooling. If the system is heating and the room is below the set temperature, the red LED fades in and out. If the temperature reaches the set point, the red LED stays solid. The same logic applies to cooling using the blue LED. If the thermostat is OFF, both LEDs are off. The LCD screen shows the date and time on the first line. The second line switches between showing the current temperature and showing the current mode with the set temperature.

Every 30 seconds, the thermostat sends a message through the UART. The message includes the current state, the room temperature, and the set temperature. This acts like its sending data to SysTec’s server. For the final product, I compared three hardware options Raspberry Pi, Microchip microcontrollers, and Freescale microcontrollers. The Raspberry Pi supports all required peripherals and has built-in Wi-Fi, making development easy. However, it uses more power and costs more. Microchip microcontrollers can support the required peripherals and offer Wi-Fi in some models. They use less power and are better suited for production devices. Freescale microcontrollers also support the needed peripherals but often require an external Wi-Fi module, which adds complexity.

For development, the Raspberry Pi is a good choice. For production, a Microchip microcontroller with integrated Wi-Fi is the best option because it balances cost, power, and performance.