Reginald True

Professor Jaipur

CS 350

27 January 2026

Lab Questions

1. At what frequency can you see the LED start to blink?

- You can start to see the LED blink at about 20 Hz or lower. When the number gets smaller, the blinking becomes easier to see.

2. At what duty cycle is the intensity of the LED perceptibly diminished from the initial 50% duty cycle?

- The LED starts to look noticeably dimmer at around 30% to 40% duty cycle. Lower numbers make the light darker

3. When changing the duty cycle of the PWM, the loop used an increment of 5 every tenth of a second. Was this perceptibly smooth? If not, what could you change to improve the visual response? Why?

- The fading was not completely smooth. It looked like small jumps. To make it smoother, you can use smaller steps, like changing by 1 instead of 5, or make the program update faster. This helps the light change more slowly and smoothly.

4. What function sets the PWM frequency for a GPIO line?

- The function GPIO.PWM(pin, frequency) sets the frequency.

5. What function sets the duty cycle for a GPIO line?

- The function pwm.ChangeDutyCycle(value) changes how bright the LED is.